Harold’s DC Circuits

Cheat Sheet

11 October 2025

Circuit Laws

| Circuit Law | Formula |  |
| --- | --- | --- |
| Kirchhoff’s Current Law (KCL) | i=1nIi=0 Iin1+Iin2=Iout1+Iout2+Iout3 |  |
|  | The total current flowing into a node or junction must equal the total current flowing out of the node or junction. (conservation of charge) |  |
| Kirchhoff’s Voltage Law (KVL) | i=1nVi=0 V1=V2+V3+V4+V5+V6 |  |
|  | The sum of all voltages around a circuit loop is equal to zero. (conservation of energy) |  |
| Ohm’s Law | V=IR |  |
| Power | P=VI P=I2R P=V2R P=Wt | An across (V) times a through (I) variable. Watt (W)=Volt (V)Ampere (A) |
| Electrical Energy | E=QV E=VIt P=I2Rt E=Pt | Joule J=Watt Wsecond s |

Components

| Element | Battery | Ground | Current Source |
| --- | --- | --- | --- |
| Component |  |  |  |
| Symbol |  |  |  |
| Denoted by | V | GND | I |
| Units | V (Volt) | 0 V | A (Ampere) |
| Series | VEQ=V1+V2 | NA | IEQ=I1+I2 |
| Parallel | V1=V2=V3 | NA | IEQ=I1+I2 |

| Element | Resistor | Capacitor | Inductor |
| --- | --- | --- | --- |
| Component |  |  |  |
| Symbol |  |  |  |
| Denoted by | R | C | L |
| Units | Ω (Ohm) | F (Farad) | H (Henry) |
| Equation | R=VRI | C=QVC | L=VLdidt |
| Series |  |  |  |
|  | RT=R1+R2+R3 | 1CT=1C1+1C2+1C3 | LT=L1+L2+L3 |
| Parallel |  |  |  |
|  | 1RT=1R1+1R2+1R3 | CT=C1+C2+C3 | 1LT=1L1+1L2+1L3 |

Electrical Symbols

Sources

- Electrical Technology (2020 Oct). Resistance, Capacitance & Inductance in Series-Parallel – Equation & Formulas.  https://www.electricaltechnology.org/2020/10/resistance-inductance-capacitance-series-parallel-formulas.html

- ForumElectrical.Com (2025).  Electrical Symbols.  https://forumelectrical.com/basic-electric-symbols/

- LibreTexts Physics (2024). 9.6: Electrical Energy and Power.  https://phys.libretexts.org/Bookshelves/University_Physics/University_Physics_(OpenStax)/University_Physics_II_-_Thermodynamics_Electricity_and_Magnetism_(OpenStax)/09%3A_Current_and_Resistance/9.06%3A_Electrical_Energy_and_Power

- Tech Web (2024).  Resonant Circuits: Resonant Frequency and Q Factor.  https://techweb.rohm.com/product/power-device/si/18332/