Harold’s Physics

Blocks and Pulleys

(Comparing Force FBDs with Impulse-Momentum)

Cheat Sheet

13 October 2026

| Blocks + Movable Pulley |  |  |
| --- | --- | --- |
| Question | Block A weighs 10 lbf. Block B weighs 3 lbf. Block B is moving downward with an initial velocity vB1=3 fts at t=0. The coefficient of kinetic friction between the horizontal plane and block A is μkA=0.15. Determine the velocity of block A at t=1 s. Assume: - The fixed pulley has negligible rotational inertia, so torque balance makes the rope tension uniform (T). - The moving pulley is supported by two rope segments (2T). - The mass of the rope is negligible. |  |
| Diagram |  |  |
| Givens | WA=10 lb WB=3 lb vB1=3 fts at t=0 | t1=0 s t2=1 s μkA=0.15 |
| Observations | g=32.2 fts2 | Fμ=Fμk= μk FN ∆t=t2-t1=1 s |
| Solution Methods | Method 1: Force FBDs | Method 2: Linear Impulse-Momentum |
| FBD Procedures | Identify the object, draw only external forces, choose axes, resolve components, and apply equations. | Identify each object’s initial momentum, net impulse, and final momentum, then show them as an L→ R box flow diagram with the vector magnitude and direction indicated per box. |
| FBDs |  |  |
| Equations | Newton’s 2nd Law: Fx=max Fy=may | Momentum: p=mv ∆p=mv2-mv1 Impulse: J=t1t2F(t) dt If a constant force, J=F ∆t=F (t2-t1) Combined: J=∆p t1t2F(t) dt=mv2-mv1 |
| Choose Signs | +x right for A | +y down for B |
| Constraints | Due to the pulley rope configuration, aA=2 aB vA=2 vB | Due to the pulley rope configuration, when B moves down, A moves right twice as fast. vA=2 vB vA1=2 vB1=2 3 fts=6 fts |
| Friction | NA=FNA=WA=10 lb Fμk=μk FNA=(0.15)(10 lb)=1.50 lb acting left |  |
| Block A (Horizontal Equation) | T-Fμk=mA aA T-Fμk=WAg aA T-1.50 lb=10 lb32.2 fts2 aA T-1.50 lb=0.3106 aA | pA1+JT-Jμk=pA2 mA vA1+T∆t-Fμk∆t=mA vA2 WAgvA1+T∆t-Fμk∆t=WAgvA2 1032.26 fts+T 1 s-1.5 1 s=1032.2vA2 |
| Block B (Vertical Equation) | WB-2 T=WBgaB 3.00 lb-2 T=3 lb32.2 fts2 aB 3.00 lb-2 T=0.0932 aB | pB1+JWB-J2T=pB2 mB vB1+WB∆t-2T∆t=mB vB2 WBgvB1+WB∆t-2T∆t=WBgvB2 332.23 fts+3 1 s-2T1 s=332.2vB2 3.2795-2T=0.09317 vB2 |
| Solve | Solve for T in the block A equation. T-1.50 lb=0.3106 (2 aB) T=1.50 lb+0.6212 aB Substitute T into the block B equation. 3.00 lb-2T=0.0932 aB 3.00 lb-2 (1.50 lb+0.6212 aB)=0.0932 aB -1.2424 aB=0.0932 aB aB = 0 T=1.50 lb aA=aB=0 fts2 | Solve for T in the block A equation. T +0.3634 =0.3106 vA2 Substitute (vA2=2 vB2) into the block A equation. T +0.3634 =0.3106 vA2 T +0.3634 =0.3106 2 vB2 T +0.3634 =0.6211 vB2 T =0.6211 vB2- 0.3634 Solve with the block B equation. 3.2795-2T=0.09317 vB2 3.2795-2 0.6211 vB2- 0.3634=0.09317 vB2 vB2=3 fts downward |
| Velocity | The blocks have zero acceleration, so their velocities remain constant. vB1=3 fts at t=0 vA1=2 (vB1)=23 fts= 6 fts vA2=vA1+aA2t =6 fts+0 fts21 s vA2=6 fts to the right | - On A, the rightward tension impulse TΔt balances the leftward friction impulse Fₖ Δt. - On B, the downward weight impulse WB Δt balances the upward rope impulse 2TΔt. - Both net impulses are zero, so both velocities remain constant. vA2=2 vB2=6 fts to the right |
| Solution | The velocity of block A at t=1 s is 6 fts to the right. |  |

Sources

- Hibbeler, R. C. (April 1, 2012).  Engineering Mechanics: Dynamics, 13th Edition, Prob. 15–32 (problem image supplied).

Acknowledgements

- Proofread with assistance from ChatGPT 6.1 Sol, https://chatgpt.com/